# May 30, 2025

# CERTIFICATE

# To whom it may concern:

This is to certify that according to the records on file in this office, the requested survey information is as follows:

| Province: NCR - MANILA, FIRST DISTRICT |  |  |
| --- | --- | --- |
| Station Name: MMA-111 |  |  |
| Order: 2nd | Accuracy Class: | Elevation: 6.389M EGM2008 |
| Island: LUZON | Barangay: LA HUERTA | Municipality: PARAÑAQUE CITY |
| PRS92 Coordinates |  |  |
| Latitude: 14° 29' 41.04714" | Longitude: 120° 59' 27.64837" | Ellipsoidal Height: 5.88400 m |
| PTM / PRS92 |  |  |
| Northing: 1602905.135 | Easting: 499031.294 | Zone: 3 |
| UTM / PRS92 |  |  |
| Northing: 1603294.4590 | Easting: 283478.2050 | Zone: 51 |
| WGS84 Coordinates |  |  |
| Latitude: 14° 29' 35.65322" | Longitude: 120° 59' 32.55140" | Ellipsoidal Height: 49.70100 m |
| Error Ellipse: |  |  |
| UTM / WGS84 |  |  |
| Northing: 1603225.98757 | Easting: 283626.44953 | Zone: 51 |

The accuracy standards reported herein (FGDC-STD-007-1998) supercedes and replace the previous accuracy standards found in FDCC 1984. Classified control points are certified as being consistent with all other points in the network, not merely those within that particular survey.

MMA-111

Is located along A. Santos Ave., on top of a 1.2 m. x 0.7 m. x 1.5 m. concrete post of the perimeter fence of Amvel Properties that serves as the gathering area of El Shaddai members. It is about 60 m. N of Airport Citimall, about 50 m. E from the station is the gate of Amvel, on the NE is a creek of about 20 m. wide and on 30 m. SW is the seafood market. Mark is the head of a 4" copper nail centered and set on top of a 30 cm. x 30 cm. cement putty, with inscription "MMA-111, 2004, NAMRIA".

# Requesting Party:

Purpose:	Reference

# OR No:

Transaction No.:	ACZ2025-0555

ENGR. NICANDRO P. PARAYNO, MSc.

OIC-Director, Mapping and Geodesy Branch